Spray Painting Booth – Inspection Checklist

| Booth: |  | Location: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

| Inspection Items | Circle appropriate response | Opportunity for Improvement | Person Responsible for Action | Action Taken | Sign off Completed |
| --- | --- | --- | --- | --- | --- |
| 3 Month Inspection (visual inspection) |  |  |  |  |  |
| Equipment |  |  |  |  |  |
| Apparatus is appropriate to area classification | Yes / No / NA |  |  |  |  |
| Any modifications comply with documentation | Yes / No / NA |  |  |  |  |
| Air flow through the booth is adequate and evenly distributed. | Yes / No / NA |  |  |  |  |
| The air quality of the compressed breathing air is not contaminated. | Yes / No / NA |  |  |  |  |
| There are no unauthorized modifications | Yes / No / NA |  |  |  |  |
| There is no undue accumulation of dust or dirt | Yes / No / NA |  |  |  |  |
| Bolts, glands, drain plugs and entry plugs are complete and tight | Yes / No / NA |  |  |  |  |
| Installation |  |  |  |  |  |
| Earthing and equipotential bonding are satisfactory | Yes / No / NA |  |  |  |  |
| There is no visible damage to equipment or cables | Yes / No / NA |  |  |  |  |

| Inspection Items | Circle appropriate response | Opportunity for Improvement | Person Responsible for Action | Action Taken | Sign off Completed |
| --- | --- | --- | --- | --- | --- |
| Environment |  |  |  |  |  |
| Apparatus is adequately protected against corrosion, moisture, vibration, excess temperature, and other adverse factors | Yes / No / NA |  |  |  |  |
| Every 12 Months (Visual and close inspection should be conducted) |  |  |  |  |  |
| Equipment |  |  |  |  |  |
| Apparatus is appropriate to area classification | Yes / No / NA |  |  |  |  |
| Any modifications comply with documentation | Yes / No / NA |  |  |  |  |
| Apparatus temperature class is correct | Yes / No / NA |  |  |  |  |
| Apparatus is clearly marked | Yes / No / NA |  |  |  |  |
| Apparatus carries the correct circuit identification | Yes / No / NA |  |  |  |  |
| There are no unauthorized modifications | Yes / No / NA |  |  |  |  |
| There is no undue accumulation of dust or dirt | Yes / No / NA |  |  |  |  |
| Accumulation of dust or dirt can result in surface temperatures higher than those permitted | Yes / No / NA |  |  |  |  |
| Apparatus is appropriate to the gas group | Yes / No / NA |  |  |  |  |
| Certificate U or X conditions are met | Yes / No / NA |  |  |  |  |
| Wiring system is appropriate | Yes / No / NA |  |  |  |  |
| Bolts, glands, drain plugs and entry plugs are complete and tight | Yes / No / NA |  |  |  |  |
| Installation |  |  |  |  |  |
| Earthing and equipotential bonding are satisfactory | Yes / No / NA |  |  |  |  |
| There is no visible damage to equipment or cables | Yes / No / NA |  |  |  |  |
| Environment |  |  |  |  |  |
| Apparatus is adequately protected against corrosion, moisture, vibration, excess temperature, and other adverse factors | Yes / No / NA |  |  |  |  |
| 3 years (Detailed Inspection Required) |  |  |  |  |  |
| Apparatus is appropriate to area classification | Yes / No / NA |  |  |  |  |
| Any modifications comply with documentation | Yes / No / NA |  |  |  |  |
| Apparatus temperature class is correct | Yes / No / NA |  |  |  |  |
| Apparatus is clearly marked | Yes / No / NA |  |  |  |  |
| Apparatus carries the correct circuit identification | Yes / No / NA |  |  |  |  |
| There are no unauthorized modifications | Yes / No / NA |  |  |  |  |
| There is no undue accumulation of dust or dirt | Yes / No / NA |  |  |  |  |
| Accumulation of dust or dirt can result in surface temperatures higher than those permitted | Yes / No / NA |  |  |  |  |
| Apparatus is appropriate to the gas group | Yes / No / NA |  |  |  |  |
| Certificate U or X conditions are met | Yes / No / NA |  |  |  |  |
| Wiring system is appropriate | Yes / No / NA |  |  |  |  |
| Bolts, glands, drain plugs and entry plugs are complete and tight | Yes / No / NA |  |  |  |  |
| Condition of enclosure gaskets is satisfactory | Yes / No / NA |  |  |  |  |
| Motor fans and couplings are not rubbing on cowls/guards | Yes / No / NA |  |  |  |  |
| Guards are correctly fixed | Yes / No / NA |  |  |  |  |
| Installation |  |  |  |  |  |
| Earthing and equipotential bonding are satisfactory | Yes / No / NA |  |  |  |  |
| Electrical connections are tight | Yes / No / NA |  |  |  |  |
| There is no visible damage to equipment or cables | Yes / No / NA |  |  |  |  |
| Environment |  |  |  |  |  |
| Apparatus is adequately protected against corrosion, moisture, vibration, excess temperature, and other adverse factors | Yes / No / NA |  |  |  |  |
| Electrical protection is satisfactory | Yes / No / NA |  |  |  |  |
| Lamp rating and type are correct A detailed inspection is necessary after replacing a lamp | Yes / No / NA |  |  |  |  |
| Installation is in compliance with documentation | Yes / No / NA |  |  |  |  |

Note: Time frames may need to be amended dependent upon the amount of usage of the booth

Comments: ______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Have all actions been communicate to person responsible		Yes/No

Signed:									Signed:

Name:						Date:			Name:						Date: