Spray Painting and Powder Coating procedure

Introduction

The purpose of this policy is to ensure that spray painting/powder coating is carried in a safe manner and to minimise the risks to the health or safety of any worker whose performs spray painting/powder coating tasks onsite or offsite. This document and relevant Safety Data Sheets (SDS) for the products used are an integral aspect of painting procedures at ${legalname}.

Scope

This procedure applies to all workers of ${legalname} and specifies the minimum requirements to be undertaken. It is expected that these procedures will be followed while working in all of the company’s paint booths, paint workshops, and principal contractor’s sites. Spray painting must be carried out in booths unless the painting consists of minor spotting or touching up operations or if the painting is carried out on plant or items that is fixed or too large to move into a booth.

Definitions

Spray painting, including electrostatic spray painting, is a process by which liquid paint is applied under pressure to an object.

Spray painting may be carried out by hand or automatically. There are several methods used to atomise the paint for spraying:

- using a conventional air compressor – air is driven across the mouth of a small outlet under pressure to draw liquid paint out of the container and produce an air-paint mist from the nozzle of the spray-gun

- airless spray painting – the paint container is pressurised pushing the paint to the nozzle where it is atomised by the spray gun, or

- electrostatic spray painting – an electric pump drives the electrostatically charged liquid paint out of the nozzle which is then applied to the object which is earthed.

Powder coating is a process by which electrostatically charged powder is applied onto an earthed object.

Triglycidylisocyanurate (TGIC) TGIC is classified as a hazardous chemical and is commonly used in powder coating activities. It is a skin sensitiser, toxic by ingestion and inhalation and genotoxic capable of causing serious eye damage.

Roles & Responsibilities

Managers/Supervisors are responsible for:

- ensuring all maintenance and inspection is carried out by a competent person as scheduled

- ensuring all workers performing spray painting/powder coating operations are suitably competent

- Ensure all staff are wearing appropriate PPE.

Workers are responsible for:

- complying with policy and procedures

- maintaining and wearing PPE as directed

Procedure

All spray painting/powder coating should be conducted within a spray booth. The role of the spray booth is to -

- carry away the over-spray,

- reduce the concentration of flammable vapour, and health hazards in the air to concentrations below the explosive limits and safe exposure levels.

Fixed items which cannot be positioned in a spray booth should be painted by other means (e.g., brushing or rollers). Authorisation of spray painting/powder coating outside a booth may occur on location, only after health, safety and environmental considerations are taken into account, and where there are means to reduce its impact to acceptable levels. Spray painting/powder coating may only be conducted by those persons who are familiar with the procedures of spray painting/powder coating and/or competently trained.

The spray booth should display a sign on every entrance door to the booth stating ‘TOXIC AND/OR FLAMMABLE VAPOURS MAY BE PRESENT’

Risk to health

During the process of painting, persons may be exposed to a variety of substances, which may have implications to their safety and health. For example, solvents and paints are formulated with a variety of ingredients. However, exposure and absorption occurs via –

- Inhalation

- Skin absorption

- Ingestion

Once within the body, the resulting dose in the body can exert an effect on various tissues and organs. This effect may be in the short term (acute effect), such as dizziness, or from repeated exposure (chronic effects), such as liver damage. The effect on health of various products can be found in their material safety data sheets, and these should be consulted by every painter before they use the product for the first time.

Personal Protective Equipment for Spray Application

Respirators

Full-face positive pressure airline respirator; This design is suitable for use in the application of all paint commonly used. This type of respirator must be used when handling two pack paints containing isocyanates. This form of respirator also includes in-built eye protection.

However, for the application of less hazardous paints (e.g. acrylic based paint) a half-face respirator with spray paint canisters (type: A1 P2 with pre-filter) may be used. Eye protection (e.g. safety glasses or goggles) is then required.

Note: In spray booths where an airline respirator is available, it should be used.

Gloves
Solvent resistant gloves made of nitrile or PVC must be worn when handling paint, solvents and during spraying. Rubber gloves are not suitable. Disposable nitrile gloves are acceptable while spraying. Re-useable gloves should be checked before use for leakage and contamination (inside) the glove.

To check for cuts and holes, simply grab and pull the cuff of the glove with both hands, flip the glove end over end two or three times to produce a good seal, then grab the cuff with one hand sealing the air within. Squeeze the glove with the other hand (the fingers should inflate), look for holes.

Overalls\coveralls
Protective clothing should be worn to avoid contamination of street clothes. Tradesmen's overalls with sleeves or disposable coveralls are suitable. Cotton or anti-static fabrics are preferred.

Hearing protection

Where noise levels are in excess of the 85 dB(A), hearing protection (earmuffs or ear plugs) should be worn. They may also be worn simply to reduce the noise to a comfortable level. When the noise level exceeds

85 dB(A) a warning sign to use protective equipment should be visible.

Footwear
Good footwear should have a non-slip chemical resistant sole and protective upper.

Maintenance
Ensure correct maintenance of PPE is conducted. This involves -

- Pre-use checks, to ensure the equipment is not damaged, leaking, and working correctly,

- Worn and correctly used, contact the manufacturer\supplier if unsure. Appropriate training shall be given before equipment is used for the first time.

- Cleaned, after use to prevent the build-up of dirt and bacteria.

- Stored correctly. For example, half face respirators should be stored in a sealed container to prevent absorption of contaminates.

- Respirator filters must be changed when appropriate.

### Refer Procedure WHSPRO-311 - PPE Procedure

### Operation of Spray Paint Booths/ Spray Painting/Powder Coating Procedures

The following are step by step procedures for spray painting –

Pre-spray procedures

Sanding

Sanding in a spray booth should only be conducted with the exhaust fan running. A respirator should also be worn. An appropriate air gun may be used to remove dust from the work piece.

Cleaning

For a good paint finish, a spray booth should be relatively free of dust. Cleaning the booth before spraying can be done by -

- Washing with water (if the area is so designed).

- Vacuuming.

- Dry sweeping.

Depending on the frequency of use and amount of dust, possible cleaning frequencies include -

- Before spraying.

- After spraying (and drying).

- Once a week.

- Equipment checks and operation

The following checks are to be conducted before each use of the spray booth -

- Ensure that the air inlet filters are clean and free from obstructions.

- Ensure all source of ignition and miscellaneous items are removed from the booth.

- Turn on the exhaust fan. Where more than one fan is present all fans must be operating.

- Turn on the inlet air fan (if present)

Note: The exhaust fan must run for a minimum of 2 minutes, before spraying can commence. This is known as the pre-spray purge cycle. During this time check -

- The air flow sensor (if present) to ensure that the minimum air flow (0.5 ms-1) is obtained.

- Where an air flow sensor is not available, use your un-gloved hand or a tissue to determine that the air is flowing efficiently.

- Ensure that the filter wall is complete and operational.

Note: If the air flow is insufficient or significantly decreases or stops, all work\spraying in booth is to stop until the problem is rectified.

- Check the compressed air supply - drain any water condensate from the system, ensure the pressure is satisfactory for the operation of the spray gun and respirator.

- Place "Spray Painting in Progress - Do Not Enter" sign on the entrance.

Note: That powder coating spray guns are usually heavier and more difficult to handle.

- Personal protective equipment

Put on your protective clothing - Overalls\coveralls,

- Air-line respirator and plug into the air supply.  Check the quality of the compressed air (by smell). An oil or burnt smell indicates problems with the compressor. Do not breathe poor quality air.

- Chemical resistant gloves

- Other items as specified above.

- ix paint

Dilute the paint with a suitable solvent into a metal can. If transferring large quantities of solvent from a drum, ensure that the drum is earthed to avoid the risk of a static spark.

- Object position

- Be aware of manual handling issues and follow the appropriate guidelines. Larger and awkward items may require two or more people to manoeuvre.

- Where possible use tables, or trolleys, to raise the working height of small items.

- When racks or shelves are used for drying, these should not impede the circulation of fresh air, exhaust of contaminated air, or the exit by persons.

- Spray gun

- Set spray gun parameters for spraying.

- Spray

- Never position yourself between the work and filters, as you will be covered in paint.

- Avoid spraying in an upward direction, as this directs spray into your breathing zone and face.

- Drying

- The booth must remain on for a post-spray cycle of at least 5 minutes. However, where possible the booth exhaust should run as long as practical, or until the items are touch dry.

- Replace the "spray painting in progress" sign with the "Paint Drying - Do Not Enter" sign.

Note: The only forms of heating allowed in a spray booth are indirect-fired heating systems, reverse cycle air-conditioning, and heat lamps. All equipment needs to be intrinsically safe to reduce the risk of fire\explosion.

- Cleaning

Clean all equipment and the mess you've made. This includes the -

- Spray gun, with a suitable solvent according to the manufacturer's instruction.

- Waste paint\solvent should be disposed of via the appropriate means - waste contractor for solvent (oil) based paints, mixed two pack paints may be allowed to harden then disposed of as solid rubbish.

- Once work with the solvents and paint is finished. Remove respirator and clean. Once dry place in a suitable location free of dust.

- Wipe gloves to remove excess wet paint. Hang out to dry. Your gloves should periodically be washed inside with soap and water and dried. If leaking, get a new pair.

- Overalls or coveralls should be allowed to dry. Cotton types should be regularly laundered. Disposable types should be thrown out once damaged or excessively contaminated.

- Restore equipment

Return equipment, personal protective equipment, paint cans and solvent to their appropriate locations. Notify manager/supervisor of any problems or malfunctions so that they may be repaired \ replaced.

### Powder Coating

- Pre-Spraying

### Surface preparation requires the use of high-risk chemicals such as chromates and hydrofluoric acid.

- Ensure that there are safe handling, storage, spill, clean-up and emergency procedures are in place.

- Equipment checks and operation

- Avoid using TGIC – free powder coatings where possible by substituting

- Conduct all powder coatings in a booth

- Powder coating spraying should be performed at least 1 meter from the stoving ovens.

- Use local exhaust ventilation when conducting powder coating activities when filling hoppers, when reclaiming powder and during clean-up

- Use automatic spray guns, feed lines and feed equipment

- Prevent unnecessary powder build-up inside powder coating booths by minimising spray gun air pressure to prevent overspray

- Interlock are to be fitted to the power supply and powder coating feed lines with the air extraction system so that if a fault develops in the ventilation system, the powder coating and power supplies are cut off

- Restricted access to spray areas

- Good personal hygiene practices and removal of dust from face and other exposed areas should be maintained and thoroughly washed

- Vacuum clothing using a vacuum with a high efficiency particulate air filter (HEPA) and not using compressed air or dry sweeping

- Never use compressed air to clean clothing or skin

- Regular inspection and cleaning of equipment

- Prevent or minimise the generation of dusts by containing the opening of powder coating packages, loading of hoppers and reclaiming of powder

- Minimise the generation of dust when filling the hopper by considering the layout of the workstation and the size of the hopper opening.

- Personal protective equipment

### Must include full protective clothing including overalls, gloves, head and eye protection and respiratory protection, selected and used in compliance with relevant Australian Standards.

### In particular:

- a powered air purifying respirator should be worn, which complies with AS/ NZS 1716-1994 Respiratory protective devices, and used in accordance with AS/NZS 1715-1994 Selection, use and maintenance of respiratory protective devices;

- the respiratory protective equipment must provide head covering to avoid dust build-up around the edges of the facemasks. A ventilated full-head covering may also be more comfortable in a hot environment;

- during manual spraying, the gun-hand must not be insulated from the gun. Either a cover sleeve must cowl the gun hand, or the palm of an insulating glove may be cut out. Operators standing outside a booth and spraying inside a booth through an aperture must wear this type of protective equipment; and

- anti-static footwear should be provided.

### Inspection and Maintenance of Spray Booths

In accordance with the Australian Standard recommendations on spray paint booths the following inspection and maintenance program is required for those booths suitable for the use of compressed air spray guns. Frequency of inspection is determined based on the amount of use of the spray booths.

The following is a recommended guide for average use of booths.

Inspections
In addition to the brief visual inspection conducted before each day’s spray painting – Ensure that the booth is cleaned on a regular basis to ensure that any overspray or residue paint is removed.

AT LEAST EVERY 3 MONTHS

In addition to the before spraying checks, a visual inspection should be conducted every three months -

- To ensure that the air flow through the booth is adequate and evenly distributed.

- The air quality of the compressed breathing air is not contaminated.

AT LEAST EVERY 12 MONTHS

Visual and close inspection should be conducted. These periodic inspections include -

- Look for loose bolts, hoses etc

- Damage to filter mounts, lights, air hoses

AT LEAST ONCE EVERY 3 YEARS

A Detailed inspection involving -

- De-energising electrical equipment.

- a close look (with tools) and opening access panels, equipment boxes etc, encompassing all equipment, systems, installation, manufacturer's guidelines.

- include earth bondage straps, and fan blades, cleaning of motors to prevent overheating.

Maintenance is to be conducted only by those who have an understanding of spray booths and is deemed as competent. A detailed inspection is required after maintenance. Modifications and repairs must meet all Australian Standard requirements.

Records
Inspection and maintenance records must be kept. It is recommended that a book (or the form attached) be used to detail information about -

- visual, close and detailed inspections

- change of filters

- any maintenance

References

Work Health & Safety Act 2012 (SA)

Work Health & Safety Regulations 2012 (SA)

Code of Practice – Spray painting and powder coating 2012

AS 4114.1:1995, Spray Painting Booths, Part 1: Design, construction, and testing.
AS 4114.2:1995, Spray Painting Booths, Part 2: Selection, installation and maintenance.

Forms

Spray Booth Inspection Maintenance Record WHSFOR-315

Authority

Signature:

Authorised by: ${officer} 	Title: ${role}

Date: ${date}