Working at Heights/Prevention of Falls

Introduction

${legalname} is committed to providing a safe work environment for its workers and preventing work injuries due to falls from height. Fall Protection is an integral part of this. This procedure describes the method for identifying the health and safety risks associated with work at height, assessing these risks and developing appropriate control measures for these risks.

Scope

This procedure applies to all ${legalname} workers who are required to undertake work at heights.

Definitions

Active Fall Protection is a system where an individual must take action to use equipment either to prevent them entering a fall hazard area or to arrest their fall.

Anchorage is the means for attaching a lanyard, lifeline or other components to a secure point.

Fall means a fall by a person from one level to another.

Individual Fall Arrest Systems are systems to arrest an accidental fall which consist of some or all of the following: Anchorage, Lifeline, Fall Arrester, Lanyard, Personal Energy Absorber, Retractable Lifeline, Rope Grabs, Wire Grabs, Rail System, Shock Absorbers - both personal and Industrial Harness.

Lanyard is a line usually used as part of a lanyard assembly to connect a fall arrest harness to an anchorage point or static line in situations where there is risk of a free fall.

Risk of a fall means a circumstance that exposes a worker while at work, or other person while at or in the vicinity of a workplace, to a risk of a fall that is reasonably likely to cause injury to the worker or other person.

Work at Height is generally, working at a height of greater than three metres above the ground or other surface which would arrest the fall (SA Only). This height may be reduced if the arresting fall surface is particularly hazardous.

Roles & Responsibilities

Managers/Supervisors:

- Identify, assess and control the risks to workers or object to fall from a height and in particular greater than 3 metres.

- Ensure workers are not working alone if they are working at heights of 3 metres or more.

- Develop a safe work method statement (SWMS) where required in conjunction with workers.

- Monitor working at heights activities and follow up any incidents or risk related.

Workers:

- Assist in the development/conduct of risk assessments and SWMS.

- Comply with all control measures, including SWMS, where required to ensure their safety and that of others.

- Report any matter or issues affecting the safety of working at heights.

Procedure

Fall hazards must be identified, in consultation with workers, for all tasks and at all locations. Tasks that commonly involve working at heights include:

- Inspection/work on bridges.

- Working on fragile surfaces i.e. skylights, cement and fibreglass sheeting roofs

- Working on roofs – slopes, brittle, unguarded.

- Working on batters.

- Climbing onto vehicles.

- Working on scaffold or EWP.

- Working on unguarded platforms, walkways and plant.

Risk Assessment

A Risk Assessment must be completed by a competent person prior to any person undertaking work at heights (unless there are known control measures which adequately control the risk).  The Risk Assessment of tasks undertaken at heights shall comply with the ${legalname} Hazard Management procedure WHSPRO-404 and also include consideration of any potential exposure of other persons and the public by ${legalname} tasks involving working at heights.  A generic risk assessment is only acceptable when fall hazards are the same and no greater, additional or different risk exposures exist. The general risk assessment should be reviewed prior to use to determine relevance and whether changes are required.

Risk controls

The most effective method of controlling the risk is to eliminate the need to work at heights. If this is not possible, reduce the risk by working down the following hierarchy of controls (using multiple controls where necessary):

- Provide fencing, handrails, barriers & overhead protection (fall protection device).

- Provide a work positioning system

- Provide an EWP.

- Provide scaffolding.

- Provide fall arrest systems and safety netting.

- Implement administrative controls - SWMS.

A combination of control measures may be required.

Work on a solid construction

Working on a solid construction provides an environment where the likelihood of a fall may be eliminated. This is the first option to be considered. ‘Solid construction’ means an area that:

- is structurally capable of supporting workers, material and any other loads applied to it

- is provided with barriers around its perimeter and around any openings from or through which a person could fall

- has an even, accessible surface and gradient

- has a safe means of entry and exit.

Temporary work platforms

Temporary work platform is considered a working platform, other than a permanently installed fixed platform, used to provide a working area for the duration of the job. The design of the platform prevents workers from falling. Temporary work platforms include scaffolds, elevating work platforms, mast climbers, workboxes, building maintenance units, portable or mobile fabricated platforms or any other platform that provides a working area and is designed to prevent a fall.

A workbox is designed to be supported by a crane, hoist, forklift truck or other mechanical device to provide an elevated work area for persons working from the box. It consists of a platform surrounded by an edge protection system and should be designed in accordance with AS 1418.17 Cranes (including hoists and winches) —Design and construction of workboxes.

A workbox fitted to a forklift must be securely attached to the forklift carriage and engineer-designed and constructed in accordance with AS 2359 Powered Industrial Trucks.

Work Positioning Systems

A work positioning system involves the use of equipment that enables a person to work supported in a harness in tension in such a way that a fall is prevented.  Work positioning systems require a high level of competency on the part of the user and supervisors to ensure safe use. Users, including supervisors, should undertake a competency-based course of training.

Fall Arrest Systems

Fall-arrest system is intended to safely stop a worker falling an uncontrolled distance and reduce the impact of the fall. Fall arrest systems include:

- Catch platforms

- Industrial safety nets

- Individual fall arrest systems

This system must only be used if it is not reasonably practicable to use higher level controls or if higher level controls might not be fully effective in preventing a fall on their own.

Individual Fall Arrest Systems

Individual fall-arrest systems consist of some or all of the following components:

- anchorages

- lifelines

- inertia reel

- lanyard of fixed length

- retractable lifelines

- rope grabs

- wire grabs

- rail system

- shock absorbers, both personal and industrial

- harness

- snap hooks (double or triple action to prevent rollout)

- karabiners (double or triple action to prevent rollout)

- rescue equipment.

Fall Arrest Systems must comply with AS/NZS 1891 series Industrial fall arrest systems and devices, safety belts and harnesses.

A local register must be established to record and schedule inspections of harnesses, lanyard assembly, body belts and any other relevant documentation.

Persons conducting review inspections on harness, lanyard assemblies, fall arrest devices and static lines must be competent in inspection procedures. Manufacturer’s requirements for such inspections must also be adhered to.

Always inspect the work area before using any fall arrest system, to make sure the potential fall path is clear. A suitable rescue procedure must be established should a person become suspended by a fall arrest system due to risk of ‘Suspension Trauma’.

The Manager and Supervisor must ensure that the fall arrest equipment is:

- The correct equipment for the job and is available.

- Inspected by competent person every 3 months and annually during full-service inspection, and records are maintained and available on site.

Persons using fall arrest equipment must:

- Never work alone while using a safety harness.

- Inspect the work area before starting.

- Check equipment before use & report any damage/wear.

- Check labels, inspection tags.

- Adjust for correct & comfortable fit.

Never use fall arrest equipment that you believe is unsafe or not right for the job. If in doubt, check with your supervisor. Individual fall-arrest systems rely on workers wearing and using them correctly, and therefore workers who will use such a system must be trained in its safe use.

Ladders

Fixed Ladders

Fixed ladders as permanent access must comply with AS 1567 & AS/NZS 1892.

If a permanent ladder is used, fall protection must be provided where a free fall in excess of 2 metres is possible. Fall protection could include a cage around the ladder, or a fall arrest system attached to the ladder.

Portable Ladders

Portable ladders used as temporary access must comply with the relevant standard of the AS 1892 series.

When using portable ladders:

- Use the correct ladder for the job.

- Use only industrial rated ladders.

- Examine ladders for defects and damage before use.

- Be adequately supported at the base.

- Extend at least one metre above the access level.

- One person on a ladder at a time with three body limbs on the ladder at all times.

- Metal ladders or wire reinforced ladders must not be used where electrical hazards exist.

- Single and extension ladders must be placed at a slope of 4:1 (ie. 4 vertical to 1 horizontal).

Electrical Safety - Overhead Utilities.

- Identify all aboveground, overhead and underground services during the work planning stage. This includes power lines and other utilities suspended on bridges and other similar structures.

- Consult with the owner of the service about their requirements for preventing damage to property and preventing injury.

- Complete a written risk assessment of all foreseeable risks associated with carrying out the work and develop control measures, eg. relocate or de-energise the apparatus prior to work, use movement inhibitors and/or machinery that will not enter the 3 metre No Go Zone.

- Verify the position of all services before work begins and make sure all required warning signs and devices are in place, always assume component are energised until written authority is gained from the access authority, and remove all warning signs and devices when services are reinstated..

- Safety observers must be utilised in an authorised person zone.

- Develop and implement a SWMS in consultation with workers involved in the work.

- Issues involving emergency conditions & overhead utilities must be included in your Emergency Evacuation Plan.

- If services have been de-activated, de-energised or disconnected, make sure that the supply authority is advised at the end of the work.

Review

Risk Assessments, SWMS, and work procedures must be reviewed and revised when:

- The original assessment is no longer valid or older than 2 years.

- Injury or illness results from incidents or accidents.

- A change is proposed at the place of work, or in work practices or procedures to which the assessment relates.

Training

Training must be recorded and include:

- Requirements of ladders, scaffolding, overhead protection & EWP.

- Fall prevention and other control devices and their use.

- Fall arrest and PPE use, care, maintenance and storage.

- Refresher training is required every 2 years

References

Work Health & Safety Act 2012 (SA)

Work Health and Safety Regulations 2012 (SA)

Code of practice - Managing the Risk of Falls at Workplaces 2011

Code of Practice - Working in the Vicinity of Overhead and Underground Electric Lines 2013

AS/NZS1891 – Industrial fall arrest systems, devices safety belts and harnesses.

AS 1418 SAA Crane Code – Part 10 Elevating Work Platforms & Pt 13 Building Maintenance Units

AS 1576 Scaffolding Parts 1-4

AS 1657 Fixed platforms, walkways, Stairways and Ladders

AS 1892 Portable Ladders

AS2626 Industrial safety Belts and Harnesses – Selection Use and Maintenance

Forms
SWMS WHSFOR-201

Risk Assessment, WHSFOR-404

Authority

Signature:

Authorised by: ${officer}	Title: ${role}

Date: ${date}