WELDING procedure

Introduction

Welding is a potentially hazardous activity and precautions are required to avoid electrocution, fire and explosion, burns, electric shock, vision damage, inhalation of poisonous gases and fumes, and exposure to intense ultraviolet radiation. This procedure provides guidance on the safe welding practices.

Scope

This procedure relates to all workers conducting welding work at or for ${legalname}. If conducting welding in a confined space workers must also follow safe confined space procedures.

Definitions

Welding is the process of permanently joining two or more materials together, usually metals, by heat or pressure or both.

Roles & Responsibilities

Managers/Supervisors are responsible for;

- Ensuring appropriate supervision, instruction and information for any workers undertaking welding

- Ensuring all hazards associated with welding are identified and controlled

- Providing appropriate and safe plant, equipment and PPE to enable workers to conduct welding safely

Workers are responsible for;

- Conducting pre welding checks to ensure all equipment is in safe condition

- Following all organisational procedures relating to welding

- Only conducting welding work if competent to do so

Procedure

Pre checks

- Check all hoses, leads and equipment for leaks or other damage

- Ensure threads and fittings to oxygen supplies are clean and oil free

- All combustibles in the area are removed

- Screens are in place and entry to area is restricted

- Ventilation/extraction are in place and working

- All PPE is in good condition and worn

Ventilation

The control of welding fumes is essential to the provision of a safe work environment for workers. Use of ventilation extraction minimises the exposure to fumes during welding. Levels of concentration (exposure standards) of welding fumes need to be assessed to determine safe levels. Results of air monitoring may assist with monitoring the level of extraction from ventilation systems. Respiratory protection may also be required.

Radiation

The effects of radiation from welding can affect both workers and passers-by. To minimise the risk control measures such as screens and partitions, signs, restricted entry, PPE including filter shades for goggles and face shields should be used.

Gas under pressure

Hazards associated with welding from compressed gas include the risk of explosion, toxicity, asphyxiation, fire, oxidation, and uncontrolled release of pressure. All cylinders must be secured and stored correctly, ensure cylinders are stored upright and secured by the use of chains, rings or similar. Flashback arrestors should be fitted at the blow pipe and to the oxygen and fuel gas regulators.

PPE

Appropriate PPE must be worn when welding;

| Equipment | Area to be protected | Protection |
| --- | --- | --- |
| Welding Helmet, shield, visor or goggles | Eyes and face | Radiation, slag, sparks, intense light, irritation and burns |
| Respirator | Respiratory | Fumes and oxides |
| Overalls, apron, clothing (fire/flame resistant) | Exposed skin | Heat, fire, burns |
| Ear plugs/muffs | Hearing | Noise |
| Gloves | Hands | Electric shock, heat, burns, fire |
| Boots | Feet | Electric shock, heat, burns, fire |

Completion of Welding

On completion of welding equipment must be made safe;

- Turn all valves and bottles off

- Open regulators and bleed lines

- Pack up the welding set and store appropriately

Maintenance of equipment

All equipment used in welding must be installed, maintained, repaired and tested properly. Gas cylinders must be regularly inspected. All electrical equipment must be properly maintained.

References

Work Health and Safety Act 2012 (SA)

Work Health and Safety Regulations 2012 (SA)

Model Code of practice – Welding Processes 2012

AS/NZS 1338: (series) Filters for eye protectors

S/NZS 2161: (series) Occupational protective gloves.

AS 1674 Set-2007 (series) - Safety in welding and allied processes

AS/NZS 3957: Light-transmitting screens and curtains for welding operations

AS/NZS 1716: Respiratory protective devices

Forms

Nil

Authority

Signature:

Authorised by: ${officer} 	Title: ${role}

Date: ${date}